Wi2Wi, Inc. Confidential

W2SG0001 (SSIII GPS Module) Development Kit Quick Start Guide

September 28, 2006

1. Turn-On the Development Kit (see Figure #1)

  - Connect J2 to computer by serial cable.

  - Supply +5VDC (provided in kit) to J1 DC Jack.

  - Connect an active antenna (provided in kit) to J3 (SMA). Place the antenna receiver close to a window for better signal reception. Jumper at JP2 should be removed if a passive antenna is used.

  - Make sure the jumper at JP5 is connected. Also make sure that the coaxial jumper from the GPS module to the Hirose connector J4 is connected.

  - Push switch SW1 (in red color) down to turn-on the development kit.

Figure #1: Development Kit Connections

2. Turn on the SiRFDemo Tool

- Start SiRFDemo application

  - Double click on “Wi2Wi GPS Tool” Folder.

  - Double click on “SiRFDemo” folder.

  - Double click on “SiRFDemo” icon.

- Data Source Setup window pops up(see Figure #2)

  - Set the serial port to which the development kit is connected (e.g. COM1).

  - Set the baud rate to 57,600.

  - Click OK.

Figure #2: Data Source Setup Window

- Setup SiRFDemo (see Figure #3 and Figure #4)

  - Connect to Data Source (click on the button shown in the figure).

  - Click on “Synchronize Protocol & Baud Rate” under “Action” menu.

- After all the steps above, you should see the following windows

  - Response View

  - Radar View

  - Map View

  - Geodetic Nav View

  - Signal View

- You should soon start seeing satellites in the “Signal View” window (see Figure #5). Since this is a cold start, it will take some time for the satellites to appear and lock.

  - If the satellites do not appear, try placing the antenna close to a window.

Figure #3: Connection to Data Source

Figure #4: Synchronizing Protocol and Baud Rate

Figure #5: SiRFDemo Output in Full Operation