# Student RF System Setup

Refer to Figure 1 at the back of this document when setting up the student systems.

- Each student system has its own packet of RF cables to interconnect the modules in the PXI system.  Remove a packet for each system from the shipping container.

- Using the short SMA to SMB cables, connect CH0 of the PXI-5441 to INPUT on the PXI-5610 and TO AWG CLK IN on the PXI-5610 to CLK IN on the PXI-5441.

- Using the SMA to BNC cable, connect OUTPUT on the PXI-5600 to CH0 of the PXI-5142.

- Using the long SMA to SMB cable, connect 10 MHz OUT on the PXI-5600 to CLK IN on the PXI-5142.

- Using the 0.45 m semi –rigid SMA to SMA cable, connect RF OUTPUT from the PXI-5610 to INPUT on the PXI-5600.

- Connect the flat panel monitor, mouse and keyboard to the PXI controller and power up the PXI chassis.

# Instructor RF System Setup

Refer to Figures 2 & 3 at the back of this document when setting up the instructor system.

- Remove the packet of cables marked for the instructor system from the shipping container.

- Using the short SMA to SMB cables, connect CH0 of the PXI-5441 to INPUT on the PXI-5610 and TO AWG CLK IN on the PXI-5610 to CLK IN on the PXI-5441.

- Using the SMA to BNC cable, connect OUTPUT on the PXI-5600 to CH0 of the PXI-5142.

- Using the long SMA to SMB cable, connect 10 MHz OUT on the PXI-5600 to CLK IN on the PXI-5142.

- Using the SMA to SMA cables in the instructor bag, connect CH1 Bank A to CH1 Bank B on the PXI-2596.

- Using the SMA to SMA cables, connect CH2 Bank A on the PXI-2596 to CH1 IN on the PXI-5690.  Connect CH2 Bank B on the PXI-2596 to CH1 OUT on the PXI-5690.

- Using an SMA to SMA cable, connect COM of Bank A on the PXI-2596 to COM on the PXI-2595.

- Using an SMA to SMA cable, connect COM of Bank B to INPUT on the PXI-5600.

- Find the two Minicircuits blue inline filters in the instructor packet.  Connect the filter labeled HPF to CH6 of Bank A on the PXI-2596.  Connect the cable from the filter to CH6 of Bank B on the PXI-2596.  Connect the filter labeled LPF to CH3 of BankA on the PXI-2596.  Connect the cable from the filter to CH3 of Bank B on the PXI-2596.

- Using the SMA to SMA cables, connect CH0 of the PXI-2595 to RF OUTPUT on the PXI-5610.  Connect CH1 of the PXI-2595 to RF OUT on the PXI-5652.

# Instructor Demonstrations

The instructor system is setup so that the various solutions and demos can be ran by simply making the proper connections with the PXI-2595 and PXI-2596 switches using the NI-Switch Soft Front Panel.  The following instructions explain which connections to make for each demo:

- Connecting the PXI-5661 directly to the PXI-5671 – This is the configuration used for most of the class.  Using the NI-Switch Soft Front Panel, connect COM to CH0 on the PXI-2595.  Connect COM A to CH1A and COM B to CH1B on the PXI-2596.

- Connecting the PXI-5661 directly to the PXI-5652 –Using the NI-Switch Soft Front Panel, connect COM to CH1 on the PXI-2595.  Connect COM A to CH1A and COM B to CH1B on the PXI-2596.

- Spectrum with Preamp Demo – This configuration connects the PXI-5661 to the PXI-5672 via the PXI-5690.  Using the NI-Switch Soft Front Panel, connect COM to CH0 on the PXI-2595.  Connect COM A to CH2A and COM B to CH2B on the PXI-2596.

- Filter Frequency Response Demo – For this demo, the software automatically configures the PXI-2596 switch.  The instructor needs to connect the PXI-5671 by using the NI-Switch Soft Front Panel to connect COM to CH0 on the PXI-2595.

# Ghosting & Tear Down of RF Equipment

This is going to be the simplest process when tearing down the equipment. The total ghosting time is twenty-two minutes per PXI Station. While the PXI Stations are ghosting, all other equipment can be torn down and packed back into the shipping case. To ghost the PXI controllers do the following:

- Reboot the PXI Controller.

- When the PXI Controller reboots you will see a menu. (You have 60 seconds to make a selection).

- Choose Symantec Ghost. (Ghost will automatically start without any other user interaction)

- While the PXI Controllers are ghosting all other equipment can be placed back into the proper shipping cases.

- Once the ghost process is done, the PXI Controller will automatically reboot back to the menu. At this point, the PXI Controller can be shut down. If the PXI Controller boots back into Windows just shut the PXI Controller down.

# Who to call if you have problems

If you come across any problems please call starting at the top of the list.

| Technician | Phone Number (from 7am to 4pm Central Time) | After Hours and Weekend Phone Number |
| --- | --- | --- |
| John Seele (1st Contact) | 512-683-6304 | 512-751-9754 |
| Jerry Frausto (2nd Contact) | 512-683-6423 |  |
| Customer Education Hotline (3rd Contact) | 512-917-2969 | 512-917-2969 |

Figure 1: Student RF system cable setup

Figure 2: Instructor RF System cable setup

Figure 3: Close up view of PXI-2595, PXI-2596 and PXI-5690